|  | Department of Electronics and Telecommunication Engineering (NBA ACCREDIATED) Antenna and Radio Wave Propagation Laboratory Academic Year 2020-2021 Odd Semester |
| --- | --- |

| Course Code | ECC603 |
| --- | --- |
| Subject Professor In-charge | Prof. Santosh Jagtap |
| Student Name | Anuj Shah |
| Roll Number | 18104B0024 |
| Class | TE EXTC |
| Division | B |
| Date of Performance | 28th April 2021 |
| Date of Submission |  |

EXPERIMENT NO.8

Design array of N elements and plot its radiation pattern

| Total (10 Marks) | Sign |
| --- | --- |

EXPERIMENT No.8

Title: Design array of N elements and plot its radiation pattern

Estimated time to complete this experiment:  02 hours

Objective: To measure gain and bandwidth of array of N elements

CO to be achieved: CO1, CO2.

Expected Outcome of Experiment:  Design of high gain antenna

Pre Lab/ Prior Concepts:  Radiation pattern, Impedance, SWR

Theory (2 Marks)

Array factor of N elements array and its properties.

Array factor:

An ESLA (eqally space linear array) consists of N equally spaced array elements, separated by the same distanced d. The phase of an array element leads the phase of the previous element by ψ=kdcosθ.

The array factor for an ESLA is:

AF(N,ψ)=m=0N-1ejmψ= 1-ej*N*ψ1-ej*12* ψ=ejN-1ψ/2* sin(N*ψ2)N*sin(ψ2)

Here are the properties of the array factor:

- Array factor represents the response of an array of isotropic elements, allowing us to treat the element and the array separately.

- The total field pattern from an array can be found by multiplying the element factor (the pattern produced by a single element) by the array factor.

- There is always a maximum at ψ=0, corresponding to θ=90o, which is called the broadside direction as it is normal to the plane of the array.

- As N increases, the width of the main lobe decreases.

- The number of sidelobes increases as N increased.

- The width of these minor lobes (in terms of ψ) is 2π/N. The width of the major lobe is twice that.

- The SLL (sidelobe level) decreases with N.

Dipole Design

Design Specification: - Frequency (f) : 300 MHz - Length of Wire (l) : 0.5λ λ =c/f=1 m l=0.5* λ=0.5 m. Where, c=Speed of light L= Length of dipole Diameter of Wire (d): d= λ/100

Array Geometry Parameter Setting

Results

- Array of 4 Elements

HBPW=80 degree Gain=9.25 dB - Array of 7 Elements

HPBW= 80 Gain= 9.57 dB

- Array of 15 Elements

Observations: Gain= 10 Log [4N (d/λ)], d=0.5 λ, Hence, Gain= 10 Log [4N (0.5)]=10 Log (2N) Sr.No; Array Elements(N); HPBW; Gain (Measured); Gain (Calculated). 1; 4; 80°; 9.25 dB; 9.03 dB. 2; 7; 80°; 9.57 dB; 11.46 dB. 3; 15; 80°; 12.1 dB; 14.77 dB.

Conclusion:

- The gain increases as the number of elements in the array increases.

- The HPBW remains the same as the number of array elements increases.